12th Man Project Brief

Pump Commissioning Test 1 – Rig Description

Process P&ID

Process Overview:

This document describes a section of the Process & Instrumentation Diagram (P&ID) of the carbon capture pilot plant at Imperial College London. The system is designed to circulate Mono-Ethylamine (MEA) through a loop to determine the maximum safe operating speed of Pump J100.

Key operational constraints:

- **No heating** from heaters H200a and H200b.

- **No cooling** from cooler C100.

- **Alternative bypass routes (yellow-highlighted streams)** exist in case of valve failure leading to leaks.

Legend:

- **Blue** – Main Stream (MEA flow path).

- **Red** – Closed Valves (preventing flow through specific paths).

- **Green** – Open Valves (allowing flow through designated paths).

- **Yellow** – Bypass Streams (alternative flow routes, numbered 1 to 11).

### MEA Stream Properties:

- **Flowrate Range**: 0 - 12,000 kg/hr

- **Temperature**: 25°C

- **Pressure**: 1 bar

- **Phase**: Liquid

- **Composition**:

- **MEA**: 15%

- **Water**: 85%

*Ideal Flow Condition*: Bypass streams (1 to 11) should have **no flow**, and the only active flow should be through the **main stream and stripper column** within the range of 0 to 12,000 kg/hr.

---

## **Detailed Process & Instrumentation Diagram (P&ID) Description:**

The **P&ID** visually represents the **carbon capture pilot plant**, illustrating **fluid flow**, **instrumentation**, and **control components** in the system.

### **Main Components & Their Functions:**

1. **Pump J100**

- Located in the **main stream**.

- Ensures the continuous circulation of **MEA** through the process.

- Monitored for maximum safe operating speed.

2. **Filter Z102**

- Positioned along the **main stream** before the fluid reaches critical components.

- Removes impurities from the circulating **MEA** to prevent equipment damage.

3. **Heaters (H200a, H200b)**

- Not operational in this process setup.

- Would otherwise provide heat to facilitate chemical reactions.

4. **Cooler C100**

- Also inactive in this setup.

- Used to maintain temperature control when required.

5. **Stripper Column**

- Core part of the system, allowing separation and gas-liquid interactions.

- Equipped with **multiple manual and needle valves** to regulate flow.

---

### **Instrumentation and Control Valves in P&ID**

The following table details the **valves** present in the system:

| **Valve ID** | **Type** | **States** | **Stream** |

|-------------|----------|------------|------------|

| V107        | Manual Globe Valve | Open/Closed | Stripper Column |

| V108        | Manual Needle Valve | Open/Closed | Stripper Column |

| V109        | Manual Globe Valve | Open/Closed | Bypass Stream 4 |

| V110        | Manual Globe Valve | Open/Closed | Main Stream |

| V111        | Manual Globe Valve | Open/Closed | Bypass Stream 5 |

| V112        | Manual Globe Valve | Open/Closed | Main Stream |

| V113a       | Manual Globe Valve | Open/Closed | Main Stream |

| V113b       | Manual Globe Valve | Open/Closed | Main Stream |

| V115        | Manual Globe Valve | Open/Closed | Bypass Stream 1 |

| V116        | Manual Globe Valve | Open/Closed | Main Stream |

**(Additional valves continue with similar descriptions for bypasses and stripper column.)**

---

### **Sensors & Monitoring Equipment**

Several transmitters are used for **pressure, flow, and level measurement**:

| **Transmitter ID** | **Type** | **Measurement Range** | **Location** |

|-------------------|----------|-----------------|------------|

| PT102 | Gauge Pressure | -1 to 4 barG | Stripper Column |

| PT103 | Gauge Pressure | -1 to 10 barG | Main Stream |

| PT104 | Gauge Pressure | -1 to 10 barG | Main Stream |

| PDT105 | Differential Pressure | 0 to 2 bar | Main Stream |

| FT103 | Electromagnetic Flow Meter | 120 - 12,000 kg/hr | Main Stream |

| FT104 | Swirl Meter | 100 - 1,600 kg/hr | Main Stream |

| LT101 | Level Transmitter | 0 - 100% | Stripper Column |

---

### **Safety & Alarm Triggers**

The following table summarizes **alarm trigger levels**:

| **Instrument ID** | **HH (High-High)** | **H (High)** | **L (Low)** | **LL (Low-Low)** | **Location** |

|------------------|-----------------|-----------|----------|-----------|------------|

| PT102 | 4.50 bar | 3.50 bar | 1.00 bar | 0.00 bar | Stripper Column |

| PT103 | 4.50 bar | 3.50 bar | 1.00 bar | 0.00 bar | Main Stream |

| FT103 | 1000 kg/hr | 800 kg/hr | 80 kg/hr | 50 kg/hr | Main Stream |

| FT104 | 1000 kg/hr | 800 kg/hr | 80 kg/hr | 50 kg/hr | Main Stream |

| LT101 | 95% | 85% | 5% | 1% | Stripper Column |

---

### **Emergency Shutoff & Relief Mechanisms**

To prevent overpressure scenarios, the system incorporates **relief and emergency valves**:

| **Valve ID** | **Type** | **State** | **Stream** |

|-------------|----------|----------|------------|

| RV101 | Relief Valve | Closed | Bypass Stream 1 |

| RV301 | Relief Valve | Closed | Bypass Stream 3 |

| RV401 | Relief Valve | Closed | Bypass Stream 2 |

| PCV122 | Pressure Control Valve | Closed | Main Stream |

Additionally, **solenoid-operated valves (SOVs)**, such as **SOV123 and SOV152**, are installed for **automated safety control**.

---

### **Summary of Process Flow**

1. **MEA enters the system** at a controlled **flow rate and pressure**.

2. The **pump (J100)** ensures continuous circulation.

3. The **filter (Z102)** removes impurities before **MEA enters the stripper column**.

4. Various **valves regulate the flow** to ensure safety and optimal performance.

5. **Instrumentation sensors continuously monitor** flow rate, pressure, and liquid levels.

6. **Emergency safety measures**, such as **relief valves and alarm triggers**, provide **automatic shutdown mechanisms** in case of abnormalities.

---

## **Final Notes**

This **detailed textual representation** of the P&ID **ensures an accurate recreation** of the process. It provides **clear documentation** of all **components, operational constraints, and safety mechanisms** used in the **carbon capture pilot plant**.