**Anomaly Overview**

The anomaly observed is that the LT101 (Level Transmitter 101) is reading 99% in the stripper column. This high reading indicates potential issues within the stripper operation that need to be addressed to restore normal conditions.

**Possible Causes of LT101 Reading 99%**

| **Possible Causes**                                                                 | **Reasoning**                                                                                                                                                                                  |

|-------------------------------------------------------------------------------------|----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------|

| 1. High level of liquid in the stripper column.                                      | The trigger of high-level switch LSHH100 (99%) suggests that the liquid level is reaching the upper limit, causing LT101 to reflect elevated levels.                                    |

| 2. Blockage in the overflow or outlet stream.                                        | If there is a blockage in the overflow or outlet, liquid will accumulate in the column, contributing to the high reading on LT101.                                                        |

| 3. Inaccurate reading of the level transmitter LT101.                               | The malfunctioning or incorrect calibration of the transmitter can lead to erroneous readings, which do not accurately reflect the actual liquid level.                                     |

| 4. Improper valve states causing backflow.                                          | Closed valves (V200 or V201) that should be open can lead to backflow, increasing the liquid levels associated with LT101.                                                                |

| 5. High flowrate through the main stream (exceeding expected range).                | A surge in flow rate might overburden the system, limiting vapor escape and elevating liquid levels in the stripper.                                                                        |

| 6. Presence of foreign objects or fouling in the stripper column.                    | Any obstructions or fouling within the stripper create flow impediments, thus raising levels detected by LT101.                                                                             |

| 7. Effects of the current operating conditions (e.g., changes in temperature or pressure).  | Changes in operating conditions can lead to increased liquid retention in the column, negatively influencing LT101 readings.                                                             |

**Chosen Cause Explanation**

Upon review by the Cause Distinguisher agent, the primary cause identified for the high reading of LT101 is a blockage in the overflow or outlet stream. This blockage is expected to be the most critical factor contributing to the elevated level observed.

**Effect of the LT101 Reading of 99%**

The effect associated with this anomaly includes a potential overflow situation, inefficient operation of the stripper process, and the risk of exceeding safety limits in the system. Such a high reading can also adversely affect downstream operations reliant on consistent stripper performance.

**Summary of the Cause**

The identified cause leading to the LT101 reading of 99% is primarily due to the blockage in the overflow or outlet stream. This blockage prevents liquid from being discharged properly, causing elevated levels within the stripper column.

**Solution to Fix the LT101 Reading**

To address the anomaly of LT101 reading 99%, the following corrective actions will be implemented:

1. **Inspect and Clear Overflow and Outlet Streams**: Identify and eliminate any blockages in the overflow or outlet streams to restore proper flow.

2. **Calibration of Level Transmitter LT101**: Inspect and calibrate LT101 to ensure accurate readings and eliminate any potential malfunctions.

3. **Check Valve Operations**: Assess the operational status of necessary valves (V200 and V201) ensuring they are functioning correctly to prevent backflow issues.

4. **Monitor Flow Rates**: Evaluate input flow rates to ensure they are within design limits, making adjustments as necessary to prevent system overload.

5. **Assess Stripper Column Operating Conditions**: Review operational parameters, adjusting temperature and pressure where needed to optimize stripper performance.

6. **Identify and Remove Foreign Objects or Fouling**: Conduct an inspection to address any foreign material or fouling within the column.

7. **Implement Monitoring After Adjustments**: Post-corrective measures, maintain continuous monitoring of LT101 to validate stabilization back to normal levels.

This comprehensive approach addresses the immediate anomaly while enhancing the reliability and performance of the stripper column, thus returning the process to its normal operating conditions.